##### Technical Guide	No. 77 (5th Edition)

##### Model	KX-TD1232CE/TD816CE/TD816PD

Title	Software information

Reference Number	20010511-1KU

1. Update

| Edition | Date of issue | Modification | Note |
| --- | --- | --- | --- |
| 1st | 11 May, 2001 | Original | - |
| 2nd | 05 June, 2001 | Add software version P241L/P341L information. | - |
| 3rd | 29 Aug., 2001 | - Add the SYS990, Area 6, bit 9 /ARS with DTMF support information. - Add software version P241LB/341LB information. | (A) |
| 4th | 09 Feb., 2002 | Add software version P241L20206C/ P341L20206C information. |  |
| 5th | 10 Jan., 2003 | Add software version P241M21129A/ P341M21129A information. |  |

2. TD1232CE Version up information

| Software Version | Modification | Mass-pro. PBX suffix |
| --- | --- | --- |
| P011W ROM 2 M RAM 256K | Original Software | 1994 A,B |
| P141B ROM 4 M RAM 256K | 1) Tariff Signal(12/16KHz) detection 2) Hotel Features (Check In/Out, Total charge by room) | June 1995 C,D |
| P141D ROM 4 M RAM 1M | EURO-ISDN : External (T0) Only （2-BRI） | July 1996 E,F |
| P141F ROM 4 M RAM 1M | 1) EURO-ISDN : External / Internal KX-TD280CE (2-BRI : Switchable T0 or S0) KX-TD286CE (6-BRI : Switchable T0 or S0) Time adjustment from ISDN network data. 2) KX-TD174 (16-SLT) | Feb., 1997 G |
| P241A ROM 8 M RAM 2 M | Modification for upgrading CPU 8 MHz to 12 MHz only (no added features) | Sep., 1997 H |
| P241B ROM 8 M RAM 2 M | 1) Digital Voice Mail Integration 2) UCD Features 3) Phantom Extension Features 4) SMDR for ISDN CLIP Service 5) E&M(TIE LINE) | Dec., 1997 I |
| P241C ROM 8 M RAM 2 M | Ditto | Apr., 1998 J |
| P241D ROM 8 M RAM 2 M | Ditto | June, 1998 K,L |
| P241E ROM 8 M RAM 2 M | 1) Primary Rate Interface 2) DECT 3) New Digital PITS & Console Point to Multi Point on CO for BRI 4) DDI table exchange 5) Ring Group 6) Remote card(9,600 bps) 7) Supported SYS990, Area 6, bit 9 /ARS with DTMF : (A) | Dec.,1998 M |
| P241G ROM 8 M RAM 2 M | 1) Support consumer DECT. 2) Toll Restriction from T75xx/T74xx function is modified 3) Current Detector function is modified 4) System reset problem is modified. (Double system) 5) System miss connection with T7540/T7440 is modified. 6) PS door phone function is modified. 7) System lock problem is modified. (Right after "system clear) 8) PS Hand over function is modified. 9) UCD function is modified. 10) VM integration function is modified. 11) Toll restriction from SLT function is modified. | May, 1999 N |
| P241H ROM 8 M RAM 2 M | 1) New DECT interface card (TD146) 2) Message Waiting Lamp Adapter for SLT KX-TD194.（include 16 extensions & 1 control port） SLT with MW-Lamp can be accommodated. 3) KX-TD184(include 4 TIE lines) 4) KX-TD185.(include 4 DID lines) 5) Check-in time and day is added to check out sheet. 6) Intercom Call and Incoming Call can be separately set FWD 7) The DISA floating extension number can be set to the IRNA forwarding destination Built-in Auto Attendant feature. Default value of DISA CO-CO is changed. 8) The mode is added though CLIP or the DDI number is continuously displayed when answer Incoming call. This can be selected with SYS-PRO 990 9) The switch day and night is more possible than the operator by the operation of one. 10) TAPI2 interface support 11) The door opening time can be selected (3sec,5sec). This can be selected with SYS-PRO 990 12) ISDN information once a day by the first call received after 3:00AM. This can be selected with SYS-PRO 990. 13) The extension user simple access to a desired party. | Dec., 1999 O |
| P241I ROM 8 M RAM 2 M | 1) Update for DISA feature When the DISA User Code is mistaken three times, the CO line is disconnected. Default value of DISA CO-CO is changed. [811] DISA User Codes: All code No. : 0001-0032 -> Blank 2) MCID function of ISDN The function of ISDN is added. Malicious Call Identification (MCID) Supplementary service (ETS 300 130) function of ISDN is added. | Mar., 2000 P |
| - | [Software] No Changed [Hardware] Power unit is changed. | Aug., 2000 Q |
| P241J ROM 8 M RAM 2 M | [Software] Please refer to the 20001221-KU(CE-5).doc. [Hardware] No Changed | Dec., 2000 R |
| P241K ROM 8 M RAM 2 M | [Software] 1) Support TD7580 2) TSP(CTI) Build136 [Hardware] No Changed | Apr., 2001 U |
| P241K 10423C IC20=D1BF IC21=BC40 | [Software] When you connect one of the following DECT systems to a PBX extension port, some time the DECT handset does not ring for the doorphone call. If you have this situation, please use this software. KXTCD950/ KXTCD951/KXTCD952/KX-TCD953/KX-TCD955/KXTCD958 KXTCD960/ KXTCD961/KXTCD962/KXTCD965 KXTCD970/ASTRA950/ASTRA951 --- Cause of this symptom --- These DECT do not support the doorphone call ring pattern (160m sec ON / 160m sec OFF) from the PBX. --- SYS[990] Area 09 bit 8: Selection of SLT ringing pattern for doorphone call --- 11111111 x1111111 0 : same as the ringing pattern for CO call (New) 1 : 160m sec ON / 160m sec OFF (default) --- Note --- This change effects for all SLTs in the system, so SLT user will not recognize the call differences between CO call and Doorphone call by hearing ring tone. [Hardware] No Changed | Special Version |
| P241L 10516A IC20=4C29 IC21=4889 | [Software] The following symptoms are modified. 1- Feature upgrade of One-Touch dialing. When you program One-Touch key on the Flexible CO button of your PT or DECT-PS, you can send to VPS DTMF signal (Exp. send DTMF “#6*800” in "165P#6*800"). 2- Support overlap dialing by internal ISDN equipment. 3- System connection trouble with DIL 1:N delayed ringing. Symptom: Some time the DIL 1:N delayed ringing does not work on the intersystem connection mode. (No trouble on the single system) [Hardware] No Changed | June 2001 V |
| P241L 10810B IC20=4B94 IC21=7785 | [Software] The following symptoms are modified. We have found the following inappropriate symptom. [Configuration] TD1232/TD816 is connecting to a voice mail system (TVP100/TVP200/TVP50). [Step and Symptom] - An outside call arrives to the VM. - The call is transferred to an extension with the FX1D disconnect sequence. - The called extension does not answer the call. - The recall timer is exceeded, the call should be back to the VM. - But when the all VM ports are occupied by another call, the call is transferred to Ext.105. (Incorrect) [Hardware] No Changed | Temporally Version |
| P241L 20206C IC20=46EA IC21=8038 | [Software] The following symptom is modified. We enable “Pay tone assignment” [423]. When a pay tone signal comes or an AOC comes from PTT, we can change the LCD display from call duration time to charge (or meter) by pressing the FWD/DND button. The Jack 1 is possible to change it by the FWD/DND button, but Jack 2-64 are not possible to change it by the FWD/DND button. [Hardware] No Changed | Temporally Version |
| P241M 21129A IC20=FB7F IC21=8038 | [Software] 1- Support the7710 PBX mode feature. We are able to use the T7710 telephone PBX mode on the TD1232/816 with this software. During PBX mode, we can dial PBX feature number by pressing A1-A8, MSG button. A1-A8 and MSG button can be assigned PBX feature number like below. We can program these settings only from PITS programming [157]. (We cannot program this setting from PC programming software.) This setting is common in system. This setting is not for each extension. Button Assigned Feature Feature (Example) Default Number(Example) A1 0 Call Operator Blank A2 102 Call Extension 102 ( Restaurant -1) Blank A3 103 Call Extension 103 ( House keeping) Blank A4 104 Call Extension 104 ( Bar) Blank A5 105 Call Extension 105 (Rent Car Office in Hotel) Blank A6 106 Call Extension 106 ( Restaurant -2) Blank A7 761 Wake Up Call setting with Additional time setting. Blank A8 107 Call Extension 107 ( Restaurant -3) Blank MSG 702 Calling back the message notification sender 702 2- Send Ring Back tone when PRI receives an incoming call -1 System Programming[990] Area09 Bit09 (1:Default Setting) 1= Not to send Ring Back tone when PRI receives an incoming call. Not to send “PROGERSS INDICATOR” in Alert Message. 0= Send Ring Back Tone when PRI receives an incoming call. Send “PROGERSS INDICATOR” in Alert Message. -2 System Programming[990]Area09 Bit10 (1:Default Setting) 1= Ring Back Tone 1 0= Ring Back Tone 2 [Hardware] No Changed [PC programming ] No Changed | Jan., 2003 Y |

3. TD816CE/PD Version up information

| Software Version | Modification | Mass-pro. PBX suffix |
| --- | --- | --- |
| P051E ROM 2 M RAM 1 M | Original Software | 1994 A-D E,F |
| P441B ROM 4 M RAM 1 M | 1) Tariff Signal(12/16KHz) detection 2) Hotel Features (Check In/Out, Total charge by room) 3) EURO-ISDN : External (T0) Only （2-BRI） | July 1996 G,H |
| P441D ROM 4 M RAM 1M | 1) EURO-ISDN : External / Internal KX-TD280CE (2-BRI : Switchable T0 or S0) KX-TD286CE (6-BRI : Switchable T0 or S0) Time adjustment from ISDN network data. 2) KX-TD174 (16-SLT) | Feb., 1997 I |
| P441F ROM 4 M RAM 1 M | Ditto | Sep., 1997 J |
| P341A ROM 8 M RAM 2 M | 1) Modification for upgrading CPU 8 MHz to 12 MHz only 2) Digital Voice Mail Integration 3) UCD Features 4) Phantom Extension Features 5) SMDR for ISDN CLIP Service 6) E&M(TIE LINE) | Dec., 1997 K |
| P341B ROM 8 M RAM 2 M | Ditto | Jan., 1998 K |
| P341C ROM 8 M RAM 2 M | Ditto | Apr., 1998 L |
| P341D ROM 8 M RAM 2 M | Ditto | June, 1998 M-O |
| P341E ROM 8 M RAM 2 M | 1) DECT 2) New Digital PITS & Console Point to Multi Point on CO for BRI 3) Point to Multi Point on CO BRI 4) DDI table exchange 5) Ring Group 6) Remote card(9,600 bps) 7) Supported SYS990, Area 6, bit 9 /ARS with DTMF : (A) | Dec.,1998 CE=P PD=E |
| P341G ROM 8 M RAM 2 M | 1) Support consumer DECT. 2) Toll Restriction from T75xx/T74xx function is modified 3) Current Detector function is modified 4) System miss connection with T7540/T7440 is modified. 5) PS door phone function is modified. 6) System lock problem is modified. (Right after "system clear) 7) PS Hand over function is modified. 8) UCD function is modified. 9) VM integration function is modified. 10) Toll restriction from SLT function is modified. | May, 1999 CE=Q PD=F |
| P341H ROM 8 M RAM 2 M | 1) New DECT interface card (TD146) 2) Message Waiting Lamp Adapter for SLT KX-TD194.（include 16 extensions & 1 control port） SLT with MW-Lamp can be accommodated. 3) KX-TD184(include 4 TIE lines) 4) KX-TD185.(include 4 DID lines) 5) Check-in time and day is added to check out sheet. 6) Intercom Call and Incoming Call can be separately set FWD 7) The DISA floating extension number can be set to the IRNA forwarding destination Built-in Auto Attendant feature. Default value of DISA CO-CO is changed. 8) The mode is added though CLIP or the DDI number is continuously displayed when answer Incoming call. This can be selected with SYS-PRO 990 9) The switch day and night is more possible than the operator by the operation of one. 10) TAPI2 interface support 11) The door opening time can be selected (3sec,5sec). This can be selected with SYS-PRO 990 12) ISDN information once a day by the first call received after 3:00AM. This can be selected with SYS-PRO 990. 13) The extension user simple access to a desired party. | Dec., 1999 CE=R |
| P341I ROM 8M RAM 2M | [Software] 1) Update for DISA feature When the DISA User Code is mistaken three times, the CO line is disconnected. Default value of DISA CO-CO is changed. [811] DISA User Codes: All code No. : 0001-0032 -> Blank 2) MCID function of ISDN The function of ISDN is added. Malicious Call Identification (MCID) Supplementary service (ETS 300 130) function of ISDN is added. [Hardware] No Changed | Mar., 2000 PD=G |
| P341J ROM 8 M RAM 2 M | [Software] Please refer to the 20001221-KU(CE-5).doc. [Hardware] No Changed | Dec., 2000 CE=S PD=H |
| P341K ROM 8 M RAM 2 M | [Software] - Support TD7580 - TSP(CTI) Build136 [Hardware] No Changed | Apr., 2001 CE=X PD=J |
| P341K 10423C IC20=1841 IC21=D68F | [Software] When you connect one of the following DECT systems to a PBX extension port, some time the DECT handset does not ring for the doorphone call. If you have this situation, please use this software. KXTCD950/ KXTCD951/KXTCD952/KX-TCD953/KX-TCD955/KXTCD958 KXTCD960/ KXTCD961/KXTCD962/KXTCD965 KXTCD970/ASTRA950/ASTRA951 --- Cause of this symptom --- These DECT do not support the doorphone call ring pattern (160m sec ON / 160m sec OFF) from the PBX. --- SYS[990] Area 09 bit 8: Selection of SLT ringing pattern for doorphone call --- 11111111 x1111111 0 : same as the ringing pattern for CO call (New) 1 : 160m sec ON / 160m sec OFF (default) --- Note --- This change effects for all SLTs in the system, so SLT user will not recognize the call differences between CO call and Doorphone call by hearing ring tone. [Hardware] No Changed | Special Version |
| P341L 10516A IC20=5512 IC21=75D5 | [Software] Same as TD1232 with P241L10516A, except the item 3. [Hardware] No Changed | June, 2001 CE=Y PD=K |
| P341L 10810B IC20=5453 IC21=CE4B | [Software] Same as TD1232 with P241L10810B. [Hardware] No Changed | Special Version |
| P341L 20206C IC20=4F4B IC21=21C2 | [Software] Same as TD1232 with P241L20206C. [Hardware] No Changed | Special Version |
| P341M 21129A IC20=499D IC21=DBE5 | [Software] Same as TD1232 with P241M21129A except the item 2. [Hardware] No Changed | Jan., 2003 CE=B PD=L |